Pre-Run Setup Options - all, any, or none can be used prior to starting a run

BOOLEAN   EX_Reset(void)

BOOLEAN   EX_SetCOMport(byte COM_Num - zero based)

BOOLEAN   EX_GetRate(double* RATE)

BOOLEAN   EX_SetRate(double RATE)

BOOLEAN	EX_GetMultiChanScanChans(BYTE* bChan0, BYTE* bChan1, BYTE* bChan2, BYTE* bChan3, BYTE* bChan4, BYTE* bChan5, BYTE* bTotalChans)

BOOLEAN	EX_SetMultiChanScanChans(BYTE bChan0, BYTE bChan1, BYTE bChan2, BYTE bChan3, BYTE bChan4, BYTE bChan5, BOOLEAN fUpdateIni)

BOOLEAN  EX_GetScanType(BYTE bScanType)

BOOLEAN  EX_SetScanType(BYTE* bScanType)

BOOLEAN	EX_SetLengthOfRunMs(UINT uiMilliseconds)

BOOLEAN	EX_SetLengthOfRunSec(UINT uiSeconds)

BOOLEAN  EX_ShowDevSetupInfo_MsgBox(void)

BOOLEAN  EX_ShowDevSetupInfo_WriteToFile(void)

BOOLEAN  EX_SetDevToDefaultSetup(void)

BOOLEAN  EX_SetDevToIniSetup(void)

BOOLEAN  EX_WriteCurrentSetupToIniFile(void)

this should be called last as the other calls may effect the interval returned by this call.

BOOLEAN	EX_GetScanInterval(double* dblCurrentInterval)

RUN (EX_Run())

BOOLEAN  EX_Run(void)

If no options are used since application was first loaded or since

a call to EX_Reset(), all default parameters built into the DLL will

be used for the RUN.

The Run will continue until either the EX_Stop function is called or

the application exits, or the "length of run" time has expired if the

run time was set to any non-zero value since the application loaded

or the last call to EX_Reset();

During-Run Options - all, any, or none can be used while running

BOOLEAN  EX_CheckRun(void)

BOOLEAN  EX_SendDigout(BYTE bDigOut)

BOOLEAN  EX_GetDigin(BYTE* bDigIn)

STOP (EX_Stop())

BOOLEAN EXs_Stop(void)

This function a run if one is currently active. It will also close any

files opened for logging data or other purposes. It will terminate

any threads, and free any allocated memory.