These are the members passed in the LabView M201_config array. The array is 50 members long for future expandability, but only 35 members are currently used. Although only one member is size of double, all are passed as double to make interraction between LabView and the DLL simpler.

==  Legend  ==================

-   Application writes this value

+  DLL writes this value

x  Needs to exist, but can be any value.

============================

0 - TXQUEUESIZE	- Transmit queue size (1024)

1 -  RXQUEUESIZE	- Receive queue size (32000)

2 -  BAUD		- selected baud rate code (0 - 5)

3 - COMPORT		- com port number 1-4

4 - SCANINTERVAL	+ scan interval in seconds. Calculated by DLL based on RATE value

5 - NUMCHANNELS 	- number of active channels for scanning

6 - AVERAGE		- remote averaging (powers of 2)

7 - TSCAN		- type of scan 0 = single channel 1 = normal 2 = normal with calibration

8 - CHAN0		+channel 0 data

9 - CHAN1		+channel 1 data

10 - CHAN2		+channel 2 data

11 - CHAN3		+channel 3 data

12 - CHAN4		+channel 4 data

13 - CHAN5		+channel 5 data

14 - SINGLECHAN	- single channel scanning channel

15 - CHANNEL		- currently selected channel (used in GetConversion)

16 - SUBCHAN		- currently selected sub channel (used in GetConversion)

17 - WORDCNT		- number of words in conversion (2 or 3)

18 - BIPOLAR		- 0 = unipolar  1 = bipolar

19 - FILTER		- 0 = 4 Hz  1 = 40 Hz  2 = 400 Hz

20 - GAIN 		- gain factor (powers of 2)

21 - RATE		- sample rate (10 - 1027 Hz)

22 - MODEREGLO	+mode register for signon

23 - MODEREGMID	+mode register for signon

24 - MODEREGHI	+mode register for signon

25 - LASTDIGINP	+last digital input value

26 - VERSION 		+Model 201 version

27 - SLEEPING		+0 = Model 201 awake  1 = Model 201 asleep

28 - STANDBY 		+0 = Model 201 out of standby  1 = Model 201 in standby

29 - ERRORTYPE	+error code returned from DLL

30 - DIGOUTWORD	- value sent to Model 201 for digital output (0-255)

31 - TIMEBASECNT	x ignored by 201 (set this to any value)

32 - RAWCOUNT	+current raw count returned from Model 201

33 - VOLTAGE		+current voltage calculated by DLL

34 - Unused		x ignored by 201 (set this to any value)